UDC 519.8

Batching on two facilities dedicated for work and rework

I.V. Gribkovskaya1, S. Kovalev2 and F. Werner3

1Molde University College, Norway,

2Ecole des Mines de Saint Etienne, Saint Etienne, France,

3Otto-von-Guericke-University Magdeburg, Germany

Planning of an imperfect production of a single product is studied. The product is assumed to be continuously divisible. There are two facilities: a main facility dedicated to the original production and a facility dedicated to re-manufacturing defective units coming from the main facility. Units fabricated on the main facility are inspected for quality in batches. After the inspection has been completed, defective units of the inspected batch are transported to the re-manufacturing facility. Given a demand for good quality units of the product and an upper bound K on the number of batches, the problem is to find a sequence of batch sizes such that the makespan, i.e., the time of the demand satisfaction, is minimized. A linear programming formulation is suggested and an O(log K) time solution algorithm is derived.

1. Problem definition. The following problem of planning the production of new and recovery of defective units of the same product is studied. A single product is manufactured on two facilities: a main facility which produces good quality units as well as defective units of the product, and a facility dedicated to re-manufacturing defective units. These facilities are called machine 1 and machine 2, respectively. It is assumed that the re-manufacturing process is perfect, i.e., machine 2 always outputs good quality units. The product is assumed to be continuously divisible. The processing of one unit of the product on machine 1 and machine 2 requires p1 and p2 time units, respectively. Product units fabricated on machine 1 are inspected for quality in batches, and the inspection of any batch requires s time units. Two types of the inspection are considered: on-line and off-line. For the on-line inspection, no unit can be produced on machine 1 while the inspection is in progress. For the off-line inspection, machine 1 is available for production during the inspection. The fraction of the defective units, which is called fraction defective in the quality control literature (see Gitlow et al. [3]), is assumed to be the same in any batch. This assumption is commonly accepted in the literature on batching work and rework processes, see the papers by Chiu et al. [2], Buscher and Lindner [1], Inderfurth et al. [4], and Inderfurth et al. [5]. Specifically, let this fraction be 1/v, which implies that the total number of units in any batch is v times the number of defective units in this batch. After the inspection has been finished, all the defective units of the same batch are sent from machine 1 to machine 2 for re-manufacturing, and this sending requires t time units. Upon arrival on machine 2, defective units become immediately available for re-manufacturing.

It can be noticed that the problem with off-line inspections can be modeled as a special case of the problem with on-line inspections. In the off-line situation, we can re-set t = t + s and s = 0 and obtain an equivalent problem with on-line inspections. Therefore, we assume without loss of generality that all the quality inspections are made on-line.

There is a demand for N good quality units. Given this demand, the number of defective units is equal to n = N/v. Due to the assumption of continuous divisibility, this number as well as the number of defective units in any batch and the size of any batch can be non-integer, which implies that the total number of batches can be infinitely large in general.

Assume that an arbitrary upper bound K on the number of batches is given. The problem is to find a production plan, which is a partition of the set of N units into at most K batches, a sequence of these batches on machine 1 and a sequence of the corresponding batches of defective units on machine 2 such that the makespan is minimized. In our case, the makespan is equal to the earliest time of the demand satisfaction. The parameters s, t, p1, p2, N, v, K are assumed to be positive integer numbers. Denote this problem as P-Time.

2. Properties. There exists an optimal solution of problem P-Time, which has the following properties: (1) machine 1 starts at time zero and has no idle time until the completion of the last unit, (2) machine 2 has no idle time since it starts processing until the completion of the last product unit, and (3) the batches of defective units are processed in the same order on machine 2 as the corresponding original batches on machine 1.

Properties (1) and (2) can be proved by shifting batches of an optimal solution to the left and to the right, respectively, so that possible idle times are removed and the makespan does not change. Property (3) can be proved by batch interchanges on machine 2. So, the problem reduces to determining the number of batches and their sizes.  An example of a solution which satisfies properties (1)-(3) is given in Fig. 1.

B1

s

B2

s

Machine 1

t                                t

Machine 2

B1

B2

Time               0

Fig 1. A solution with batches B1 and B2, which satisfies properties (1)-(3).

It can be noticed that the problem with on-line inspections, whose time is a linear function of the batch size, can be reduced to the same problem with a constant inspection time. Assume that the inspection of any batch B requires s(B) = s + |B|a time units. An equivalent problem is obtained by re-setting p1 = p1 + a and s(B)= s.

3. Motivation. Problem P-Time was observed in a make-to-order porcelain toy and kitchenware production. At the final stage of this production, every item is cleaned and inspected for non-smooth spots by a human operator. If non-smooth spots are detected, the corresponding item is polished to get the required quality. The polishing can be done by the same or another operator, which depends on the urgency of the order. If the order completion time is important, defects are always removed by another operator. The reason is that if one operator performs cleaning, inspection and polishing, then too much time is spent on setting up between the different jobs. Items enter the final stage, move between the two operators, and leave the final stage in containers placed on a moving belt to prevent their accidental falling down. This situation can be modeled as problem P-Time, in which p1 is equal to the cleaning and inspection time of an item, s is equal to the removal time of two containers with good quality and defective items from the working area of the first operator, t is equal to the transportation time of the container with defective items, and p2 is equal to the processing time of an item by the second operator. In the future, the quality inspection can be performed automatically by an intelligent visual device. In this case, cleaned items will be placed on a pallet and moved to the inspection zone. The visual device will scan items on the same pallet (within s time units) and detected defectives will be moved (within t time units) to the polishing zone.

4. A linear programming formulation. The following notations are used.

Parameters:

s – the quality inspection time of a batch;

t –  the transportation time of the defective units of the same batch

from machine 1 to machine 2;

pi – the processing time for any unit of a product on machine i, i = 1,2;

N – the demand for good quality units;

1/v – the fraction of defective units in any batch;

n –  the total number of defective units, n = N/v;

K –  the upper bound on the number of batches.

Decision variables:

k – the number of batches;

xi – the size of a batch sequenced i-th, i = 1,…,k.

Objective function to be minimized:

T(k)(x) – the makespan of a solution x = (x1,…,xk).

Let T* denote the optimal solution value.

Consider a solution of problem P-Time with exactly k (non-empty) batches, 1 ≤ k ≤ K. It is completely characterized by the sequence of batch sizes on machine 2, x = (x1,…,xk). The corresponding makespan is calculated as follows:

T(k)(x) = max1≤j≤k{Mj(x)}= max1≤j≤k{(vp1xi+s)+t+p2xi}

= t + np2 + max1 ≤ j ≤ k {js + (vp1 – p2)xi+ vp1xj},

where Mj(x) = t + np2 + js + (vp1 – p2)xi+ vp1xj ) is a lower bound for the makespan. Problem P-Time reduces to K problems of minimizing T(k)(x), subject to xj= n and xj > 0, j = 1,…,k. By introducing additional variables yk = T(k)(x), k = 1,…,K, problem P-Time reduces to the following K linear programming problems, denoted as LP(k), k = 1,…,K.

Problem LP(k):

min yk, subject to

Mj(x) = t + np2 + js + (vp1 – p2)xi+ vp1xj ≤ yk, j = 1,…,k,

xi=n,

xj ≥ 0, j = 1,…,k.

Let yLPk denote the minimum solution value for problem LP(k). Since problems LP(k) contain constraints xj ≥ 0, j = 1,…,k, that are weaker than the required constraints xj > 0, j = 1,…,k, we have T* ≥ TLP = min 1 ≤ k ≤ K {yLPk}. Let xLP = (xLP1,…,xLPk) be an optimal solution of problem LP(k) with the value yLPk = TLP, and let I = {i | xLPi}. It is easy to see that a solution x* = (x*1,…,x*r), r = k - |I|, which is obtained from xLP by removing the zero components, has the makespan value T(x*) = TLP, and therefore, it is an optimal solution for problem P-Time.

The above reduction shows that problem P-Time can be solved by standard linear programming techniques. Since there are K problems LP(k) with k + 1 variables and k + 1 constraints, each of which can be solved by a polynomial time algorithm of Karmarkar [6] in O(k3.5) time, problem P-Time can actually be solved in O(k4.5) time.

5. An O(logK) solution approach. Our solution approach for problem P-Time is based on the following theorem, which is a generalization of a similar theorem in Potts and Baker [7] to the case of non-zero inspection and transportation times, s and t.

Theorem 1. For any optimal solution x = (x1,…,xk), k ∈{1,…,K}, of problem P-Time, the following equations are satisfied:

Mj(x) = t + np2 + js + (vp1 – p2)xi+ vp1xj ) = T*, j = 1,…,k.

Having proved theorem 1, we can state that problem P-Time is solvable in O(logK) time. As the input of problem P-Time includes numbers s, t, p1, p2, N, v, K, a polynomial algorithm for this problem must be polynomial in log(max{s,t,p1,p2,N,v,K}). Therefore, the O(logK)-time algorithm is polynomial for problem P-Time.

Conclusion. In this presentation, a specific batching problem which appears in an imperfect production environment is described. Problem properties are established and a motivation is provided.  A linear programming formulation is suggested, which has led to an O(k3.5)-time solution algorithm. Furthermore, properties of an optimal solution are used to derive an O(log K)-time solution algorithm.

## References

1. Buscher, U. Optimizimg a production system with rework and equal sized batch shipments / U. Buscher, G. Lindner // Computers and Operations Research. – 2007. – Vol. 34, No. 2. – P. 515-535.

2. Chiu, H.N. Optimal production lot sizing with rework, scrap rate, and service level constraint / H.N. Chiu, C.-K. Ting, Y.-S.P. Chiu // Mathematical and Computer Modelling. – 2007. – Vol. 46. – P. 535-549.

3. Gitlow, H. Tools and methods for the improvement of quality / H. Gitlow, S. Gitlow, A. Oppenheim, R. Oppenheim  // Boston: Irwin. – 1989. – 603 p.

4. Inderfurth, K. Batching work and rework processes with limited deterioration of reworkables / K. Inderfurth, A. Janiak, M.Y. Kovalyov, F. Werner // Computers and Operations Research. – 2006. – Vol. 33. – P. 1595-1605.

5. Inderfurth, K. Cost minimizing scheduling of work and rework processes on a single facility under deterioration of reworkables / K. Inderfurth, M.Y. Kovalyov, C.T. Ng, F. Werner // International Journal of Production Economics. – 2007. – Vol. 105. – P. 345-356.

6. Karmarkar, N. New polynomial-time algorithm for linear programming / N. Karmarkar // Combinatorica. – 1984. – Vol. 4, No. 4. – P. 373-395.

7. Potts, C.N. Flow shop scheduling with lot streaming / C.N. Potts, K.R. Baker // Operations research letters. – 1989. – Vol. 8, No. 6. – P. 297-303.